# Table S1. Complete parameter provenance

## Table S1A. Values, ranges, context, and model use

| Parameter | Estimate | Lower | Upper | Unit | Context | Used as |
| --- | --- | --- | --- | --- | --- | --- |
| airport_free_walking_speed | 1.3 | 1.0 | 1.6 | m/s | airport terminal pedestrians | sensitivity range |
| pedestrian_startup_delay | 0.39 | 0.2 | 0.8 | s | controlled single-file stop/start experiment | distribution anchor |
| pedestrian_reaction_delay | 0.48 | 0.2 | 1.0 | s | controlled single-file stop/start experiment | distribution anchor |
| pedestrian_slowdown_time | 0.58 | 0.2 | 1.2 | s | controlled single-file stop/start experiment | distribution anchor |
| queue_space_per_passenger |  | 0.6 | 1.39 | m2/passenger | security checkpoint single queue | sensitivity constraint |
| accessible_route_clear_width | 0.915 | 0.915 |  | m | accessible walking surface | hard geometry constraint |
| accessible_turning_diameter | 1.525 | 1.525 |  | m | wheelchair circular turning space | hard geometry constraint |
| luggage_footprint_multiplier | 1.25 | 1.0 | 1.6 | ratio | luggage-laden airport pedestrians | engineering sensitivity |
| screening_service_rate | 180.0 | 120.0 | 240.0 | passengers/hour/lane | generic checkpoint lane | engineering sensitivity |
| service_time_cv | 0.65 | 0.35 | 1.2 | ratio | generic stochastic lane service | sensitivity parameter |
| luggage_prevalence | 0.65 | 0.0 | 1.0 | proportion | generic airport passengers | sensitivity parameter |
| reduced_mobility_prevalence | 0.05 | 0.0 | 0.2 | proportion | generic passenger heterogeneity | sensitivity parameter |
| gate_release_delay | 0.75 | 0.0 | 3.0 | s | batch gate or visual release | sensitivity parameter |
| body_footprint | 0.2 | 0.16 | 0.25 | m2/person | generic schematic passenger footprint | sensitivity parameter |
| luggage_footprint_per_bag | 0.12 | 0.08 | 0.2 | m2/bag | generic carried or wheeled luggage footprint | sensitivity parameter |
| static_clearance | 0.35 | 0.25 | 0.55 | m2/person | stationary holding clearance | sensitivity parameter |
| moving_clearance | 0.55 | 0.4 | 0.85 | m2/person | movement clearance before luggage turning envelope | sensitivity parameter |
| row_spacing | 1.2 | 0.9 | 1.8 | m | parallel stationary rows | sensitivity parameter |
| gate_width | 1.2 | 0.915 | 2.0 | m | release opening | geometry constraint and sensitivity |
| barrier_footprint | 8.0 | 0.0 | 12.0 | m2 | generic checkpoint holding layout | sensitivity parameter |
| row_orientation | 0.0 | 0.0 | 45.0 | degrees | parallel stationary rows | screened design factor |
| release_compliance | 0.98 | 0.8 | 1.0 | proportion | stationary row control | sensitivity parameter |

## Table S1B. Sources and evidence notes

| Parameter | Source | DOI or URL | Access date | Evidence quality | Notes |
| --- | --- | --- | --- | --- | --- |
| airport_free_walking_speed | Young 1999 | 10.3141/1674-03 | 2026-09-24 | peer-reviewed airport observations | Exact subgroup estimates require full-text extraction; broad range avoids false precision |
| pedestrian_startup_delay | Tavana et al. 2024 | 10.1016/j.tbs.2023.100659 | 2026-09-24 | peer-reviewed motion capture | Reported mean is 0.39 s; sensitivity bounds are explicit modeling assumptions |
| pedestrian_reaction_delay | Tavana et al. 2024 | 10.1016/j.tbs.2023.100659 | 2026-09-24 | peer-reviewed motion capture | Reported mean is 0.48 s; not interpreted physiologically |
| pedestrian_slowdown_time | Tavana et al. 2024 | 10.1016/j.tbs.2023.100659 | 2026-09-24 | peer-reviewed motion capture | Reported mean is 0.58 s; broad sensitivity bounds |
| queue_space_per_passenger | PANYNJ terminal planning guideline citing IATA ADRM 9th edition | https://crp.trb.org/acrp0715/wp-content/themes/acrp-child/documents/115/original/Terminal_Planning_Guidelines.pdf | 2026-09-24 | authoritative but legacy 2004 values | Converted from 6.5-15.0 ft2/passenger; not asserted as current IATA 12th-edition requirement |
| accessible_route_clear_width | 2010 ADA Standards section 403.5.1 | https://www.ada.gov/assets/pdfs/2010-design-standards.pdf | 2026-09-24 | official enforceable US minimum | Local requirements may be stricter |
| accessible_turning_diameter | 2010 ADA Standards section 304.3.1 | https://www.ada.gov/assets/pdfs/2010-design-standards.pdf | 2026-09-24 | official enforceable US minimum | Equivalent T-shaped space is permitted by the standard |
| luggage_footprint_multiplier | Davis and Braaksma 1988 | 10.1016/0191-2607(88)90014-3 | 2026-09-24 | peer-reviewed airport study | Source supports class-dependent space requirements; numerical range is deliberately treated as assumption pending full-text extraction |
| screening_service_rate | FAA AC 150/5360-13A and PANYNJ terminal planning guidance | https://www.faa.gov/documentLibrary/media/Advisory_Circular/AC-150-5360-13A-Airport-Terminal-Planning.pdf | 2026-09-24 | authoritative guidance says airport-specific | Not an empirical estimate attributed to the guidance |
| service_time_cv | explicit model assumption |  | 2026-09-24 | assumption | Gamma service distribution; stress-tested with long-inspection mixture |
| luggage_prevalence | explicit model assumption informed by luggage literature | 10.1016/0191-2607(88)90014-3 | 2026-09-24 | assumption | No airport population prevalence claim |
| reduced_mobility_prevalence | explicit model assumption |  | 2026-09-24 | assumption | No clinical prevalence claim |
| gate_release_delay | explicit model assumption |  | 2026-09-24 | assumption | Includes signal perception and gate actuation |
| body_footprint | explicit model assumption |  | 2026-09-24 | assumption | Not a crowd-safety threshold |
| luggage_footprint_per_bag | explicit model assumption informed by Davis and Braaksma 1988 | 10.1016/0191-2607(88)90014-3 | 2026-09-24 | assumption | Source supports luggage-dependent space needs but these numerical bounds are not presented as extracted empirical estimates |
| static_clearance | explicit model assumption |  | 2026-09-24 | assumption | Varied broadly because no checkpoint-specific empirical estimate was retained |
| moving_clearance | explicit model assumption |  | 2026-09-24 | assumption | Varied independently from static clearance |
| row_spacing | explicit model assumption |  | 2026-09-24 | assumption | Must also satisfy accessible route and turning constraints |
| gate_width | 2010 ADA minimum plus engineering sensitivity | https://www.ada.gov/assets/pdfs/2010-design-standards.pdf | 2026-09-24 | mixed official minimum and assumption | 0.915 m is the retained route-width minimum; larger values are engineering cases |
| barrier_footprint | explicit model assumption |  | 2026-09-24 | assumption | Layout cases must report the selected footprint |
| row_orientation | explicit model assumption |  | 2026-09-24 | assumption | Prespecified discrete screen at 0 15 30 and 45 degrees |
| release_compliance | explicit model assumption |  | 2026-09-24 | assumption | Noncompliance is actively falsified |